# LoRaWAN based Water Level Monitoring

# Protocol Document

Uplink Messages: (Device -> Thingsboard)

- Water Level Data Payload:

# A10000000D1A850000000000

| - A1 | -> | Type of payload |
| --- | --- | --- |
| - 0000000D | -> | Packet Number (0000000D16 = 1310) |
| - 1A85 | -> | Water Level (1A8516 = 678910 = 67.89%) |
| - 00 | -> | Reserved_1 |
| - 00 | -> | Reserved_2 |
| - 00 | -> | Reserved_3 |
| - 00 | -> | Reserved_4 |
| - 00 | -> | Reserved_5 |

Note: The water level value is stored as an integer and must be divided by 100 to determine the actual percentage.

- Current Device Configurations Retrieval ACK / Device Configuration Update ACK:

# A20000000D0E1000000000

| - A2 | -> | Type of payload |
| --- | --- | --- |
| - 0000000D | -> | Packet Number (0000000D16 = 1310) |
| - 0E10 | -> | Upload Interval (sec) (0E1016 = 360010) |
| - 00 | -> | Reserved_1 |
| - 00 | -> | Reserved_2 |
| - 00 | -> | Reserved_3 |
| - 00 | -> | Reserved_4 |

Downlink Messages: (Thingsboard -> Device)

  - Update Device Configurations:

# B1070800000000

| - B1 | -> | Type of payload |
| --- | --- | --- |
| - 0708 | -> | Upload Interval (sec) (070816 = 180010) |
| - 00 | -> | Reserved_1 |
| - 00 | -> | Reserved_2 |
| - 00 | -> | Reserved_3 |
| - 00 | -> | Reserved_4 |

  - Fetch Device Configurations:

# B2

    - B2	->	Type of payload